SIMATS ENGINEERING

CO3- ASSESSMENT TOOL 2                            SIMULATION TASK

COURSE NAME: DIGITAL SIGNAL PROCESSING
COURSE CODE: ECA09


6. Truncation vs Rounding Error Spectrum

Simulate the frequency-domain characteristics of quantization errors produced by truncation

and rounding in a 20-tap FIR filter and compare their error spectra. (25) (BL5) (WK4)

Given Data:

● FIR Filter Length = 20 taps

● Input Signal = White Gaussian Noise

● Quantization = 8 bits

● FFT Length = 1024

Tasks:

1. Implement truncation-based quantization.

2. Implement rounding-based quantization.

3. Obtain error signals.

4. Compute FFT of error signals.

5. Plot error spectra.

6. Compare spectral characteristics.

MATLAB Code:

clc; clear; close all;

%% Parameters

N = 20;                % FIR filter length

bits = 8;              % Quantization bits

L = 1024;              % FFT length

%% Generate White Gaussian Noise

x = randn(1, L);

%% FIR Filter (Random coefficients)

h = randn(1, N);

%% Filter Output (Original)

y = filter(h, 1, x);

%% Quantization Step Size

q = 2^(-bits);

%% --- Truncation Quantization ---

y_trunc = floor(y / q) * q;

%% --- Rounding Quantization ---

y_round = round(y / q) * q;

%% Error Signals

e_trunc = y - y_trunc;

e_round = y - y_round;

%% FFT of Errors

E_trunc = fft(e_trunc, L);

E_round = fft(e_round, L);

%% Magnitude Spectrum

E_trunc_mag = abs(E_trunc);

E_round_mag = abs(E_round);

%% Frequency Axis

f = linspace(0, 1, L);

%% Plot

figure;

subplot(2,1,1);

plot(f, 20*log10(E_trunc_mag));

title('Error Spectrum (Truncation)');

xlabel('Normalized Frequency');

ylabel('Magnitude (dB)');

grid on;

subplot(2,1,2);

plot(f, 20*log10(E_round_mag));

title('Error Spectrum (Rounding)');

xlabel('Normalized Frequency');

ylabel('Magnitude (dB)');

grid on;

Output :